Steel Sheets

# XL Satin Powder-coated Steel Sheet

The heatbed has embedded high Curie temperature magnets. These magnets are able to firmly hold removable spring steel sheets.

The Satin sheet further expands our line-up of print sheets. In terms of printing properties, the Satin sheet sits between the smooth and textured variants. Its powder-coated surface with lightly textured matte finish offers an optimal level of adhesion, especially when printing PLA and PETG materials. It is fully compatible with a wide range of common and less-common filaments, such as ABS, ASA, FlexFill 98A, Polycarbonate and more. A vast majority of materials can be printed without a separation layer (e.g. glue).

Polycarbonate - prints better on satin (compared to smooth and textured sheets) - when printing with PC often, glue stick layer is recommended to prevent possible damage to the sheet.

Find more details about the spring steel sheets in our knowledgebase.

WARNING:

Never clean the Satin powder-coated spring steel sheet with acetone! Doing so will create microfractures in the texture, which will make the surface deteriorate over time. Learn more about printing on the Satin powder-coated steel sheet in our knowledge base.

ATTRIBUTES

|  | + Suitable for both PLA and PETG |  | - Acetone causes the surface to quickly deteriorate |
| --- | --- | --- | --- |
|  | + Lightly textured surface finish |  | - Prints from ASA, PC Blend require a brim / skirt / shield around the print |
|  | + No need for glue stick for PETG |  | - DO NOT use sharp metal objects (e.g. a spatula) to remove the print! |
|  | + Wide range of supported filaments, including advanced materials, for instance PC Blend |  |  |
|  | + Easy maintenance and good adhesion qualities |  |  |

# XL Textured Powder-coated Steel Sheet

OVERVIEW:

The heatbed has embedded high Curie temperature magnets. These magnets are able to firmly hold removable spring steel sheets.

Powder coating directly on metal makes it very hard to damage this build plate. If a heated nozzle crashed into it, the metal can dissipate the heat. Powder coating also gives the surface a distinct textured look that will be visible on your prints. The textured surface hides most of the mechanical damage caused by tools. Only the top of the small bumps can be scratched, which will not imprint on the bottom of the model. This surface is used on our print farm, look at the printed parts you have on your printer for reference.

Find more details about the spring steel sheets in our knowledgebase.

WARNING:

Never clean the Textured PEI Powder-coated spring steel sheet with acetone! Doing so will create microfractures in the PEI texture, which will make the surface deteriorate over time. Learn more about printing on the Textured PEI Powder coated steel sheet on our Blog.

ATTRIBUTES

|  | Scratchproof, hard to damage |  | PLA prints with a small contact area may need a brim |
| --- | --- | --- | --- |
|  | Transfers texture to prints |  | PLA prints with a huge footprint may warp |
|  | Prints automatically detach after sheet cools down |  | Never clean with acetone |
|  | No need for glue stick for PETG |  |  |

# XL Smooth PEI Print Sheet

### OVERVIEW:

The heatbed has embedded high Curie temperature magnets. These magnets are able to firmly hold removable spring steel sheets.

These magnets are able to firmly hold removable spring steel sheets. It is the best solution for printing PLA and it has great adhesion with almost all materials. The bottom of your prints will be smooth.

You can leave small marks on the print surface with your nozzle or tools, they will typically be shinier than the rest. It does not affect functionality or adhesion. However, if you want to have the same surface look on the whole printbed you can resurface it. The easiest way is to take the hard side of a dry kitchen sponge and wipe the affected area with a circular motion gently a few times.

Find more details about the spring steel sheets in our knowledgebase.

Tiny dots may appear on the PEI-Ultem steel sheet surface. These can slightly affect the finished print bottom surface. However, they do not impact the printing process, reliability in any way. We do not consider these dots a reason for a warranty claim.

ATTRIBUTES

|  | + Perfect for PLA |  | - Do not clean with IPA before PETG - Release agent might be necessary |
| --- | --- | --- | --- |
|  | + Great adhesion with almost all materials |  | - You must apply glue stick when printing Flex |
|  | + Smooth bottom of prints |  |  |
|  | + Tiny details will stick great |  |  |